DBACT THEOREM 1

Proofs and Supporting Results

Boundary-measure-induced Local-CVT practical stability near a selected nondegenerate local branch

| Branch | DBACT-research-v3 |
| --- | --- |
| Baseline HEAD | 64e1905e092d19fd9dcbf0e77fbd39f55f0562ba |
| Proof status | Conditional local theorem; A8 interval certificate not supplied |
| Wolfram | Connector exact checks passed; 9 local scripts exit 0 |
| Regression | 470 passed, 3 skipped |

CLAIM BOUNDARY

No global PL. No formal caging. No finite-time phase completion. No unconditional safety or cooperative-transport success.

Contents and dependency chain

1  Scope and frozen claim

2  Notation and assumptions

3  Lemma 1 - density consistency

4  Lemma 2 - centroid perturbation

5  Proposition 1 - ideal gradient

6  Proposition 2 - aggregate disturbance

7  Proposition 3 - reference rate and local regularity

8  Theorem 1 - practical stability

9  Corollaries

10 Limits and evidence status

Appendix A  Constant ledger

Appendix B  Wolfram manifest

boundary/map certificate
        |
        v
Lemma 1 -> Lemma 2 -> Proposition 2
                |             /
                v            /
        Proposition 1 ------+
                |
        Proposition 3 (A8 conditional)
                |
                v
             Theorem 1 -> corollaries

The chain contains no SEARCH, caging, contact, CBF-invariance, or transport-success node.

1. Scope and frozen claim

Bounded local-map disagreement and bounded reference variation imply practical stability of the boundary-induced Local-CVT allocation dynamics around a nondegenerate local CVT branch.

The system is first order, p_i' = u_i. The nominal ENCLOSE law is u_i^nom = -k_c(p_i - c_hat_i). The ideal objective uses a saturated performance function; dropping saturation would introduce an uncancelled moving-boundary term.

F_R(r) = min(r^2, R_l^2)
H*(P,t) = integral_D min_i F_R(||q-p_i||) phi*(q,t) dq

Deliberate exclusions

- no global CVT optimality or arbitrary-shape global convergence;

- no finite-time SEARCH-to-HOLD guarantee;

- no formal geometric or topological caging;

- no contact-transport theorem or unconditional cooperative-transport success;

- no safety claim outside feasibility and deviation contracts;

- no uniform Hessian bound for an unspecified local branch.

2. Notation and assumptions

D is a fixed rectangle; P stacks N planar robot positions. Gamma(t) is the true boundary with perimeter at most P_max. The true offset target pushes arc length forward to nu*_t; phi* is its convolution with the unnormalized Gaussian plus phi_0.

K_sigma(x) = exp(-||x||^2/(2 sigma^2))
nu*_t = xi(.,t)_# ds
phi*(q,t) = phi_0 + (K_sigma * nu*_t)(q)
V_i^l = Vor_i(P) intersect D intersect B(p_i,R_l)

||K_sigma||_1 = 2 pi sigma^2
||grad K_sigma||_1 = sqrt(2) pi^(3/2) sigma
||grad K_sigma||_inf = exp(-1/2)/sigma

A1-A10

- A1. D is rectangular and the trajectory stays in fixed compact tube X.

- A2. Separation is at least d_s>0 and R_comm >= 2 R_l.

- A3. The closed piecewise-C2 boundary is rigid in version 1 and P_Gamma <= P_max.

- A4. phi_0>0, sigma>0, and the stated unnormalized Gaussian is used.

- A5. A conservative map certificate supplies every boundary/normal/offset/voxel/weight/tail bound.

- A6. LocalCVT uses midpoint cells and eta_m < m_min.

- A7. Density biases are disabled or explicitly included in delta.

- A8. A symmetry-reduced selected branch has uniform reduced Hessian >= m_H I (conditional here).

- A9. Degenerate tie/threshold sets have measure zero; the solution remains on the branch tube.

- A10. ZOH, numerical, tracking, and safety deviations have stacked-norm certificates.

3. Lemma 1 - boundary/map to density

Introduce an exact discrete intermediate measure. The comparison is performed after convolution; no total-variation convergence between a continuous arc-length measure and a Dirac measure is asserted.

nu*_t -> nu*_{h,t} = sum_k Delta s_k delta_{xi*_k}
      -> nu_hat_{i,t} = sum_k w_hat_ik delta_{xi_hat_ik}

epsilon_xi = epsilon_b + epsilon_d + 2 d_max sin(epsilon_n/2)
E_w = L_miss + M_spur + sum_matched |w_hat_k - Delta s_k|

epsilon_phi = L_K,1 P_max (epsilon_xi + sqrt(2)v + L_T h_s/2)
              + 2 pi sigma^2 E_w + epsilon_tail          (L1)
epsilon_tail <= pi R_l^2 M_drop exp(-s_sigma^2/2)

Proof

Unit normals separated by angle theta differ by 2 sin(theta/2); the triangle inequality and a square-voxel diagonal give the target error. The fundamental theorem of calculus along a translation segment, followed by Tonelli, gives ||K(.-a)-K(.-b)||_1 <= ||grad K||_1 ||a-b||.

On each arc, the midpoint curve approximation costs at most L_K,1 L_T Delta s_k^2/2. Summing uses sum Delta s_k^2 <= h_s sum Delta s_k <= h_s P_max. Matched target displacement costs at most L_K,1 P_max(epsilon_xi+sqrt(2)v), and total missing/spurious/weight discrepancy costs ||K||_1 E_w.

Every discarded Gaussian source is at least s_sigma sigma from every query in the disk. Its pointwise contribution is therefore at most exp(-s_sigma^2/2); summing dropped mass and integrating over at most pi R_l^2 proves the tail. The triangle inequality proves (L1).

Evidence boundary

Wolfram exactly checked the kernel integrals and symbolically checked the angle and step identities. Pushforward, translation, curve quadrature, measure decomposition, and tail control are the human analytic proof above.

4. Lemma 2 - mass and centroid perturbation

r0 = min(R_l, d_s/2, W_D/2, H_D/2)
m_min = phi_0 pi r0^2 / 4 > 0
m_max = pi R_l^2 (phi_0 + P_max)
phi_hat_max = phi_0 + P_max + E_w
L_hat_phi <= (P_max + E_w) exp(-1/2)/sigma

r_h = h/sqrt(2),  A_V <= pi R_l^2,  P_V <= 2 pi R_l
eta_m = L_hat_phi r_h A_V + phi_hat_max(2 P_V r_h + pi r_h^2)
eta_a = (phi_hat_max+R_l L_hat_phi)r_h A_V
        + R_l phi_hat_max(2 P_V r_h + pi r_h^2)
epsilon_q = (eta_a + R_l eta_m)/(m_min - eta_m)

||c_hat_i-c*_i|| <= 2 R_l epsilon_phi_i/m_min
                       + epsilon_q_i + epsilon_G_i      (L2)

Proof

Separation puts B(p_i,d_s/2) in robot i's Voronoi cell. The intersection with the local disk and rectangle contains at least a quarter disk of radius r0; phi*>=phi0 gives m_min. Kernel boundedness and disk area give the ideal m_max. Lemma 1 bounds mapped total weight by P_max+E_w; differentiating its convolution gives the displayed mapped-density bounds.

A midpoint cell entirely inside the exact convex cell contributes at most L_hat_phi r_h times area. Boundary-cut cells occupy a parallel strip of area at most 2P_V r_h + pi r_h^2. This proves eta_m. Repeating the argument for the shifted first moment a=integral(q-p_i)phi dq proves eta_a.

Because m_hat >= m_min-eta_m>0, the ratio identity gives epsilon_q. Lemma 1 changes continuum mass by at most epsilon_phi and shifted moment by at most R_l epsilon_phi; both densities retain the base-density floor, giving 2R_l epsilon_phi/m_min. Add the separately certified geometric-cell term. The denominator gate fails closed.

5. Proposition 1 - ideal gradient and descent

grad_{p_i} H* = 2 m*_i (p_i-c*_i)                 (P1-a)
u*_i = -k_c(p_i-c*_i) = -[k_c/(2m*_i)] grad_{p_i}H*

Inside V_i^l the active integrand derivative is 2(p_i-q); outside the disk it is zero because the cost is saturated. On a shared Voronoi face, competing minimum values agree and the two fluxes cancel. On r=R_l, the inner and outer costs both equal R_l^2, so the circle flux cancels. The workspace boundary is fixed and the exceptional tie/threshold sets have measure zero. Only the volume derivative remains, proving P1-a.

A(P,t) = diag_i(k_c/(2m*_i) I_2) >= a0 I
a0 = k_c/(2m_max)
Hdot* = -grad H*^T A grad H* <= -a0 ||grad H*||^2

If a point in B(p_i,R_l) is won by robot j, then ||p_j-p_i|| <= 2R_l <= R_comm. Thus the communication set contains every local competitor and the cell is exact. The Wolfram fixed-square gradient check is only a sanity example; it is not the shape-derivative proof.

6. Proposition 2 - aggregate disturbance

Pdot = -A(P,t) grad H*(P,t) + d(t)
||d(t)|| <= delta
delta = k_c sqrt(sum_i epsilon_c_i^2) + epsilon_zoh + epsilon_num
        + epsilon_trk + epsilon_sf                         (P2)

Lemma 2 bounds the centroid component, and the stacked triangle inequality adds the remaining deviations. Production commands use a per-agent Euclidean speed cap, so ||u_i^sf-u_i^nom||<=2u_max and epsilon_sf<=2sqrt(N)u_max. A componentwise box cap would require 2sqrt(2N)u_max. The certificate records the chosen convention.

7. Proposition 3 - reference rate and regularity

xi = b + d n,   xi_dot = b_dot + d_dot n + d n_dot
v_xi,max = v_O_bar + omega_O_bar(R_O+d_max+L_d)
nu_phi <= L_K,1 P_max v_xi,max
nu_H = 2 R_l^2 nu_phi                                  (P3)

The rigid-twist bound controls every target speed; convolution translation controls the density rate. Since 0<=min_i F_R<=R_l^2, |partial_t H*|<=R_l^2 nu_phi. On fixed compact X, |min f-min g|<=sup|f-g| supplies the same bound for H_min*, producing nu_H. Deformation, perimeter change, or split/merge events require a new measure-rate assumption.

M5 decision

1/2 ||grad H*||^2 >= mu (H*-H_min*)
H*-H_min* >= (alpha/2) dist^2(P,C*(t))

A uniform reduced Hessian lower bound on a convex symmetry-reduced chart implies both relations with mu=alpha=m_H (or smaller constants). The implication is proved. Its premise is conditional: no selected branch, gauge, cell combinatorics, or complete parameter tube was supplied for validated interval subdivision. No floating-point eigenvalue is used as proof.

8. Theorem 1 - local practical stability

V(P,t) = H*(P,t)-H_min*(t) >= 0
a0 = k_c/(2m_max),   lambda_H = a0 mu
B = delta^2/(2a0) + nu_H

V(t) <= exp[-lambda_H(t-t0)] V(t0)
        + (B/lambda_H)(1-exp[-lambda_H(t-t0)])           (T1-a)
limsup dist(P(t),C*(t)) <= sqrt(2B/(alpha lambda_H))     (T1-b)

Proof - every inequality named

Vdot = grad H*^T Pdot + partial_t H* - H_min*_dot
 <= -grad H*^T A grad H* + ||grad H*|| ||d|| + nu_H
      [Propositions 1 and 3]
 <= -a0 ||grad H*||^2 + delta ||grad H*|| + nu_H
      [Proposition 2 and A >= a0 I]
 <= -(a0/2)||grad H*||^2 + delta^2/(2a0) + nu_H
      [Young]
 <= -a0 mu V + B = -lambda_H V + B
      [local PL]

The scalar comparison lemma yields T1-a. Hence limsup V<=B/lambda_H. Local quadratic growth gives dist(P,C*)<=sqrt(2V/alpha), which yields T1-b. The infinite-time limit is asserted only if the trajectory continues to satisfy the regular-tube premises.

Status

The analytic implication is complete, and Wolfram verifies the scalar algebra. The result remains conditional on A5, A8-A10 and the run/tube certificates. It is local, not global.

9. Corollaries

Frozen perfect information

delta=0, nu_H=0  =>  B=0
V(t) <= exp[-lambda_H(t-t0)] V(t0),  dist(P,C*) -> 0

The generator and unit tests require exactly zero forcing and zero ultimate radius in this case.

Sampled data

P_{k+1}=P_k+dt[-A_k g_k+d_k],  a0 I<=A_k<=a1 I
abar=a0-(L_H a1^2 dt)/2
C_delta=delta(1+L_H a1 dt)
B_d=C_delta^2/(2abar)+(L_H/2)delta^2 dt+nu_H

dt < 2a0/(L_H a1^2),  0 < abar mu dt <= 1
V_{k+1} <= (1-abar mu dt)V_k+B_d dt
limsup dist(P_k,C*_k) <= sqrt(2B_d/(alpha abar mu))

The descent lemma first gives dt[-a0 x^2+delta x]+(L_H dt^2/2)(a1 x+delta)^2+nu_H dt. Expansion, Young, and local PL give the recursion. The geometric series and quadratic growth give the ultimate radius. Wolfram checked the expansion; randomized positive-parameter tests independently compare the direct one-step expression with the certified upper bound.

10. Limits and evidence status

| Result | Status | Limiting condition |
| --- | --- | --- |
| Lemma 1 | PROVED + CAS | A5 map bounds and tail separation |
| Lemma 2 | PROVED + CAS | rectangle, midpoint gate, exact-cell contract |
| Proposition 1 | PROVED | regular tie/threshold sets; saturated cost |
| Proposition 2 | PROVED | declared stacked disturbance components |
| Proposition 3 | CONDITIONAL | rate proved; A8 branch coercivity assumed |
| Theorem 1 | CONDITIONAL | complete implication under A1-A10 |
| Sampled corollary | PROVED | explicit step gates |

The illustrative certificate validates the dependency wiring and gates. Its numeric radius is intentionally reported as conservative and may be vacuous; it is not experimental evidence and is not used to claim transport performance.

Evidence classes and runtime gates

- PROVED: the analytic implication is written in this supplement.

- VERIFIED: Wolfram reproduced only the identified integral or algebraic subclaim.

- CONDITIONAL: a named branch, tube, map bound, or runtime deviation certificate remains required.

- NUMERIC-SANITY: debugging evidence only; no such result is used as proof here.

- Fail-closed gates: locality, positive arc length, midpoint rule, eta_m<m_min, disabled unbudgeted biases, and sampled step size.

Claim audit

The supplement deliberately contains no global PL statement, no formal caging claim, no finite-time phase-completion result, and no unconditional transport or safety guarantee.

Appendix A. Constant ledger

| Constant | Value | Unit | Status |
| --- | --- | --- | --- |
| kernel_l1 | 0.251327 | m^2 | verified |
| kernel_grad_l1 | 1.57496 | m | verified |
| kernel_grad_linf | 3.03265 | 1/m | verified |
| epsilon_xi | 0.00449999 | m | conditional |
| epsilon_phi | 0.106034 | m^3 | proved |
| m_min | 5.02655 | m^3 | proved |
| m_max | 34.5827 | m^3 | proved |
| grid_spacing | 0.000390625 | m | verified |
| eta_m | 0.0598959 | m^3 | proved |
| eta_a | 0.0574695 | m^4 | proved |
| epsilon_q | 0.0212188 | m | proved |
| epsilon_c | 0.0549703 | m | proved |
| delta | 0.156932 | m/s | conditional |
| nu_phi | 0.464928 | m^3/s | proved |
| nu_H | 0.595108 | m^5/s | proved |
| a0 | 0.0130123 | 1/(s*m^3) | proved |
| lambda_H | 0.000650615 | 1/s | conditional |
| B | 1.54143 | m^5/s | proved |
| ultimate_radius | 307.843 | m | conditional |
| sampled_Bd | 1.54617 | m^5/s | proved |
| sampled_ultimate_radius | 308.792 | m | conditional |

(epsilon_b, epsilon_n, epsilon_d, v, h_s, E_w, M_drop)
  -> epsilon_phi -> epsilon_c -> delta -> B
  -> sqrt(2B/(alpha lambda_H))

Appendix B. Wolfram verification manifest

Connector: Wolfram (version not exposed). Local runtime: WolframScript 1.14.0. All nine included scripts executed with exit code 0. Exact connector outputs and assumptions are archived in theory/wolfram/results.

| ID | Target | Evidence | Verdict |
| --- | --- | --- | --- |
| W-L1-01/02 | Gaussian L1 integrals | EXACT | PASS |
| W-L1-03 | gradient radial maximum | SYMBOLIC | PASS |
| W-L1-04/05 | normal/step identities | SYMBOLIC | PASS |
| W-L2-01/02 | ratio, monotonicity, mass | SYMBOLIC | PASS |
| W-P1-01 | integrand and fixed-cell check | SYMBOLIC | PASS |
| W-P3-01 | reference-rate algebra | SYMBOLIC | PASS |
| W-T1-01 | Young and comparison ODE | SYMBOLIC | PASS |
| W-C1-01 | sampled-data expansion | SYMBOLIC | PASS |
| W-M5-01 | generic quadratic implication | SYMBOLIC | PASS |
| W-M5-02 | actual branch Hessian box | INTERVAL | CONDITIONAL |

Evidence rule: exact/symbolic output supports only the stated integral or algebra. Interval evidence would need to cover the complete selected tube. Numeric sanity is debugging evidence only. There are no numeric-sanity entries in this package.

Reference

J. Cortes, S. Martinez, and F. Bullo, 'Spatially-distributed coverage optimization and control with limited-range interactions,' ESAIM: COCV, 11(4):691-719, 2005. DOI 10.1051/cocv:2005024.